Example of incorrectly decoded numbers in GSM_mobileAPplication

No.     Time                       Source                Dest                  Protocol Info

210 2004-12-20 12:23:47.779645 mgw1                  hlr1                  GSM MAP  insertSubscriberDataSACK TC-BEGIN stid(ba2e1dfa) insertSubscriberData

Frame 210 (214 bytes on wire, 214 bytes captured)

Ethernet II, Src: 00:90:8f:03:52:15, Dst: 00:03:ba:01:b4:98

Internet Protocol, Src Addr: mgw1 (192.168.1.4), Dst Addr: hlr1 (192.168.1.3)

Stream Control Transmission Protocol

MTP 2 User Adaptation Layer

Message Transfer Part Level 3

Signalling Connection Control Part

Transaction Capabilities Application Part

GSM_MobileAPplication

invoke

invokeId: invokeid (0)

invokeid: 1

invokeCmd: insertSubscriberData (7)

imsi: 12082300000039F8

provisionedSS: 1 item

Item: callBarringInfo (1)

callBarringInfo

ss-Code: baoc - barring of all outgoing calls (146)

callBarringFeatureList: 2 items

Item

basicService: teleservice (3)

teleservice: allSpeechTransmissionServices (16)

ss-Status: 04

Item

basicService: teleservice (3)

teleservice: allShortMessageServices (32)

ss-Status: 04

0000  00 03 ba 01 b4 98 00 90 8f 03 52 15 08 00 45 00   ..........R...E.

0010  00 c8 48 c4 00 00 40 84 ad 96 c0 a8 01 04 c0 a8   ..H...@.........

0020  01 03 0b 58 0b 58 7b 80 67 70 a9 3b 1e 46 03 00   ...X.X{.gp.;.F..

0030  00 10 7b 82 ec a0 00 00 29 40 00 00 00 00 00 03   ..{.....)@......

0040  00 98 00 02 50 61 00 03 70 fa 00 00 00 02 01 00   ....Pa..p.......

0050  06 01 00 00 00 88 00 01 00 08 00 00 00 35 00 0e   .............5..

0060  00 78 43 52 48 ea b1 09 00 03 0d 17 0a 12 07 00   .xCRH...........

0070  12 04 53 64 05 00 21 0a 12 06 00 11 04 83 67 33   ..Sd..!.......g3

0080  20 00 53 62 51 48 04 fa 1d 2e ba 6b 1a 28 18 06    .SbQH.....k.(..

0090  07 00 11 86 05 01 01 01 a0 0d 60 0b a1 09 06 07   ..........`.....

00a0  04 00 00 01 00 10 02 6c 2d a1 2b 02 01 01 02 01   .......l-.+.....

00b0  07 30 23 80 08 12 08 23 00 00 00 39 f8 a7 17 a1   .0#....#...9....

00c0  15 04 01 92 30 10 30 06 83 01 10 84 01 04 30 06   ....0.0.......0.

00d0  83 01 20 84 01 04                                 .. ...

The IMSI should be 218032000000938

Example of BER error

No.     Time                       Source                Dest                  Protocol Info

35 2004-12-20 12:23:07.623809 hlr1                  mgw1                  GSM MAP  sendAuthenticationInfoTC-BEGIN stid(229) sendAuthenticationInfo

Frame 35 (178 bytes on wire, 178 bytes captured)

Ethernet II, Src: 00:03:ba:01:b4:98, Dst: 00:90:8f:03:52:15

Internet Protocol, Src Addr: hlr1 (192.168.1.3), Dst Addr: mgw1 (192.168.1.4)

Stream Control Transmission Protocol

MTP 2 User Adaptation Layer

Message Transfer Part Level 3

Signalling Connection Control Part

Transaction Capabilities Application Part

GSM_MobileAPplication

invoke

invokeId: invokeid (0)

invokeid: 1

invokeCmd: sendAuthenticationInfo (56)

BER Error: Sequence expected but Class:0 PC:0 Tag:4 was unexpected

0000  00 90 8f 03 52 15 00 03 ba 01 b4 98 08 00 45 00   ....R.........E.

0010  00 a4 04 b4 40 00 40 84 b1 ca c0 a8 01 03 c0 a8   ....@.@.........

0020  01 04 0b 58 0b 58 00 00 75 66 90 ae 17 18 00 03   ...X.X..uf......

0030  00 82 7b 82 ec 68 00 03 70 ca 00 00 00 02 01 00   ..{..h..p.......

0040  06 01 00 00 00 72 00 01 00 08 00 00 00 35 00 0e   .....r.......5..

0050  00 62 43 aa 87 14 f2 09 80 03 10 1a 0d 12 06 00   .bC.............

0060  11 04 72 04 02 20 04 95 93 08 0a 12 07 00 12 04   ..r.. ..........

0070  53 64 05 00 21 3a 62 38 48 02 29 02 6b 1e 28 1c   Sd..!:b8H.).k.(.

0080  06 07 00 11 86 05 01 01 01 a0 11 60 0f 80 02 07   ...........`....

0090  80 a1 09 06 07 04 00 00 01 00 0e 02 6c 12 a1 10   ............l...

00a0  02 01 01 02 01 38 04 08 72 04 02 20 04 95 93 f8   .....8..r.. ....

00b0  00 00                                             ..